TUTORIAL: Digital I/O

LED Toggle with Push-Button

Name:  Yechan Kim						ID: 22100153

I. Introduction

We will learn how to control digital I/O of GPIOs of the MCU board to turn on/off an LED with a push-button input. The LED should be turned on when the button is pressed.

The objectives of this lab are to learn how to

Read and configure registers of digital GPIO of MCU

Program firmware to control digital input/output pins

Create your own functions for GPIOs

Hardware

NUCLEO -F411RE

Software

Keil uVision IDE, CMSIS, EC_HAL

Documentation

STM32 Reference Manual

II. Basics of GPIO IN

A. GPIO Digital Out Register

List GPIO registers for this LAB

| Type | Register Name | Description |
| --- | --- | --- |
| GPIO | MODER | Mode: Input |
|  | PUPDR | Pull-Up Pull-Down: |
|  | IDR | Input Data Register |

Schematic

Process of GPIOx register initiation

0. Enable Peripheral Clock (AHB1ENR) 1. Configure as Digital Input (MODER) 2. Configure pull-up/down resistors (PUPDR) 3. Read Data (IDR)

III. Tutorial

A. Register Configuration

1. GPIO: Digital In - Pin Initialization & Read PushButton

Port C Pin 13 / Input // Pull-Up

use #define BUTTON_PIN 13

- MODER: Input (MODER5[1:0]=[0 0])

GPIOC ->MODER &= ~(3<<( BUTTON_PIN *2)); // clear bits at both [26] and [27]

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |
| Logic | Bitwise AND |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mask | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Value | x | x | x | x | 0 | 0 | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |

- PUPDR: pull-up  (PUPDR13[1:0]=[0 1])

GPIOC->PUPDR &= ~(3<<( BUTTON_PIN *2)); GPIOC->PUPDR ¦= (1<<( BUTTON_PIN *2));

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |  |
| Logic | Bitwise OR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mask | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Value | x | x | x | x | 0 | 1 | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |  |

- IDR: Read Push-Button Value  (IDR13= 1)

GPIOC->IDR =(IDR>>BUTTON_PIN) & (1);

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |  |
| Logic | Bitwise OR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mask | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Value | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | 1 | x | x | x | x | x | x | x | x | x | x | x | x | x |  |

B. Programming

Preparation

Open the program ‘Keil uVision5’ and create a new project named as ‘TU_GPIO_Digital_InOut_LED_Button’.

- “../repos/EC/Tutorial/TU_GPIO_Digital_InOut_LED_Button/”

Create a new item called ‘TU_GPIO_Digital_InOut_LED_Button.c’

Copy and paste from the source ‘TU_GPIO_Digital_InOut_LED_Button_student.c’.

Include provided ecRCC2.h and ecRCC2.c library files in your project.

Exercise

Fill in the empty spaces in the code. Then, compile(F7) and flash(F8) the source code

on the MCU board.

Solution

Appendix

See here for MCU resources

- Pin Configuration of NUCLE-F401RE

- LED/Button Circuit Diagram

Result

Source Code

/**

******************************************************************************

* @author  SSSLAB

* @Mod       2024-09-06 by YKKIM

* @brief   Embedded Controller:  Tutorial Digital In

*                    - Turn on LED LD2 while Button B1 is pressing

*

******************************************************************************

*/

// GPIO Mode             : Input(00), Output(01), AlterFunc(10), Analog(11, reset)

// GPIO Speed            : Low speed (00), Medium speed (01), Fast speed (10), High speed (11)

// GPIO Output Type: Output push-pull (0, reset), Output open drain (1)

// GPIO Push-Pull    : No pull-up, pull-down (00), Pull-up (01), Pull-down (10), Reserved (11)

#include "stm32f4xx.h"

#include "ecRCC2.h"

#define LED_PIN    5        //LD2

#define BUTTON_PIN 13

int main(void) {

/* Part 1. RCC GPIOA Register Setting */

RCC_HSI_init();

RCC_GPIOA_enable();

RCC_GPIOC_enable();

/* Part 2. GPIO Register Setting for OUTPUT*/

// GPIO Mode Register

GPIOA->MODER &=  ~(3<<(2*LED_PIN));                                     // Clear '00' for Pin 5

GPIOA->MODER |=  (1<<(2*LED_PIN));                                      // Set '01' for Pin 5

// GPIO Output Type Register

GPIOA->OTYPER &= ~(1<<LED_PIN);                                     // 0:Push-Pull

// GPIO Pull-Up/Pull-Down Register

GPIOC->PUPDR  &=     ~(3<<( LED_PIN *2));                                       // 00: none

// GPIO Output Speed Register

GPIOA->OSPEEDR &=  ~(3<<LED_PIN);

GPIOA->OSPEEDR |=  (2<<LED_PIN);                                     //10:Fast Speed

/* Part 3. GPIO Register Setting for INPUT*/

// GPIO Mode Register

GPIOC->MODER &= ~(3<<(BUTTON_PIN*2));                                       // 00: Input

// GPIO Pull-Up/Pull-Down Register

GPIOC->PUPDR &= ~(3<<(BUTTON_PIN*2));

GPIOC->PUPDR |= (2<<(BUTTON_PIN*2));                                    // 10: Pull-down

/* Button Value Initialization */

unsigned int btVal=0;

/* Part 4. Deal loop  */

while(1){

//Read bit value of Button

btVal= (GPIOC->IDR) & (1UL << BUTTON_PIN);

if(btVal == 0)

GPIOA->ODR |= (1UL << LED_PIN);

else

GPIOA->ODR &= ~(1UL << LED_PIN);

}

}

Screen Shot