TUTORIAL: Digital Out

Name: Yechan Kim 							ID: 22100153

I. Introduction

In this lab, we will learn how to control the Digital Output of GPIOs of the MCU board to turn on/off an LED.

The objectives of this lab are to learn how to

Read and configure registers of digital GPIO of MCU

Program firmware to control digital input/output pins

Hardware

NUCLEO -F411RE

Software

Keil uVision IDE, CMSIS, EC_HAL

Documentation

STM32 Reference Manual

II. Basics of GPIO Out

A. Bit Operation

Fill in the blanks. You should write the answer in both hexa-decimal and binary number.

|  | Bit operation | Description | Answer |
| --- | --- | --- | --- |
|  | 0xA & 0x2 | AND | 0x2 |
|  | 0x3 ¦ 0xC | OR | 0xF |
|  | ~0x7 | NOT | 0x8 |
|  | 0xF ^ 0x5 | XOR(Toggle) | 0xA |
|  | 0x1 << 3 | Shift left | 0x8 |
|  | 0xC >> 2 | Shift right | 0x3 |
|  | 0x11 ¦= 1<<3 | Set bit | 0x19 |
|  | 0xFF &= ~1<<4 | Clear bit | 0xEF |
|  | val=0x0F &1<<3 | Read bit | 0x0F |

B. GPIO Digital Out Register

List GPIO registers for this LAB

| Type | Register Name | Description |
| --- | --- | --- |
| GPIO | GPIOx_MODER | Mode: Output/Input/Analog |
|  | GPIOx_OTYPER | Output Type: Opendrain/Push-Pull |
|  | GPIOx_OSPEEDR | Output Speed: |
|  | GPIOx_PUPDR | Pull-Up Pull-Down: |
|  | GPIOx_ODR | Output Data Register |

Schematic

Process of GPIOx register initiation

0. Enable Peripheral Clock (AHB1ENR) 1. Configure as Digital Output (MODER) 2. Configure pull-up/down resistors (PUPDR) 3. For Output: Configure Output Type (OTYPE) 4. For Output: Configure Output Speed (OSPEEDR) 5. Output Data (ODR)

III. Tutorial

A. Register Configuration

1. GPIO: Digital Out - Pin Initialization & Set LED

Port A Pin 5 / Output / Push-Pull / No Pull-Up & No Pull-Down / Default Speed

use #define LED_PIN 5

- MODER: Output  (MODER5[1:0]=01)

GPIOA->MODER & = ~(3<<( LED_PIN *2)); // clear bits at both [10] and [11] GPIOA->MODER ¦ = 1<<(LED_PIN *2); // set bit at [10]

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |
| Logic | Bitwise OR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mask | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Value | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | 0 | 1 | x | x | x | x | x | x | x | x | x | x |

- OTYPER: Push-Pull  (OT5=0)

GPIOA->OTYPER &= ~(1<< LED_PIN)

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |
| Logic | Bitwise AND |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mask | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| Value | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | 0 | x | x | x | x | x |

- PUPDR: no pull-up no pull-down  (PUPDR5[1:0]=00)

GPIOA->PUPDR &= ~(3<<( LED_PIN *2)); GPIOA->PUPDR ¦= 0<< ( LED_PIN *2)

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |
| Logic | Bitwise OR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mask | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Value | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | 0 | 0 | x | x | x | x | x | x | x | x | x | x |

- ODR: Set LED (ODR5=1)

GPIOA->ODR ¦= 1<<LED_PIN;

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x |  |
| Logic | Bitwise OR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mask | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |  |
| Value | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | x | 1 | x | x | x | x | x |  |

B. Programming

Preparation

Open the program ‘Keil uVision5’ and create a new project.

“ /repos/EC/Tutorial/TU_GPIO_Digital_Out_LED/”

Name the project as ‘TU_GPIO_Digital_Out_LED’.

Create a new item(file) and name it as ‘TU_GPIO_Digital_Out_LED.c’

Copy and paste from the source code ‘TU_GPIO_Digital_Out_LED_student.c’.

Download ecRCC_student.h and ecRCC_student.c. Then, change the file names as ecRCC2.h and ecRCC2.c.

Save them in \lib folder:  “repos/EC/include”

Exercise

This is an example code of turning ON LED without button input. Do not worry if you do not understand what this code means yet. You will learn one by one in the following few weeks.

Fill in the empty spaces in the code. Then, compile(F7) and flash(F8) the source code onto the MCU board. Verify the program by checking if the LED is turned ON.

Solution

Appendix

See here for MCU resources

- Pin Configuration of NUCLE-F401RE

- LED/Button Circuit Diagram

Results

Source Code

/**

******************************************************************************

* @author  SSSLAB

* @Mod       2024-8-23 by YKKIM

* @brief   Embedded Controller:  Tutorial Digital Out

*                  - Turn on LED LD2

*

******************************************************************************

*/

// GPIO Mode             : Input(00), Output(01), AlterFunc(10), Analog(11, reset)

// GPIO Speed            : Low speed (00), Medium speed (01), Fast speed (10), High speed (11)

// GPIO Output Type: Output push-pull (0, reset), Output open drain (1)

// GPIO Push-Pull    : No pull-up, pull-down (00), Pull-up (01), Pull-down (10), Reserved (11)

#include "stm32f4xx.h"

#include "ecRCC2.h"

//#define LED_PIN    PA_5       //LD2

#define LED_PIN 5

int main(void) {

/* Part 1. RCC GPIOA Register Setting */

RCC_HSI_init();

RCC_GPIOA_enable();

/* Part 2. GPIO Register Setting */

// GPIO Mode Register

GPIOA->MODER &=  ~(3<<( LED_PIN *2));                                           // Clear '00' for Pin 5

GPIOA->MODER |=  1<<(LED_PIN *2);                                           // Set '01' for Pin 5

// GPIO Output Type Register

GPIOA->OTYPER &=~(1<<LED_PIN) ;     Clear '00'

//

// 0:Push-Pull

// GPIO Pull-Up/Pull-Down Register

GPIOA->PUPDR  &= ~(3<<( LED_PIN *2));                                           // 00: none

// GPIO Output Speed Register

GPIOA->OSPEEDR &=  ~(3<<( LED_PIN *2));

GPIOA->OSPEEDR |= 2<<( LED_PIN *2);                                     //10:Fast Speed

// Dead loop & program hangs here

while(1){

GPIOA->ODR |= (1 << LED_PIN);  // Set LED_PIN = H, others=L

}

return 0;

}

Screen Shot